Key parameters were set to high-end values

- Chemical concentration in biosolids is set to the 95% percentile (Appendix C)

- Biosolids Application Rate: For the crop and pasture scenarios, the default dry application rate is 10 MT dry weight/ha-application applied once a year on April 1st for 40 years (Appendix E)

- Consumption Rates are set to the 90th percentile based on the most current EPA guidance on exposure: the 2011 Exposure Factors Handbook (U.S. EPA, 2011)

The BST also requires a variety of environmental setting parameters, which were set to central tendency values (e.g., median, where available) rather than high-end values to reduce the likelihood of results being overly conservative.

Solids content of Land-applied biosolids are assumed 40%. The range is 10% - 50%.