Simulated Leaching of PFAS from Land-Applied Municipal Biosolids at an Agricultural Field Site.

Jeff A.K. Silva1*, Jiří Šimůnek2, Jennifer L. Guelfo3, John E. McCray4

1Arclight Research & Consulting, Golden CO, USA; jsilva@arclightrc.com

2Department of Environmental Sciences, University of California Riverside, Riverside, CA.

3Department of Civil, Environmental, and Construction Engineering, Texas Tech University,

Lubbock, TX.

4Civil & Environmental Engineering Department, Hydrologic Science & Engineering Program,

Colorado School of Mines, Golden, CO.

*Corresponding author email: jsilva@arclightrc.com,

15 Pages

Contents

Supplemental Information	i

List of Figures	ii

List of Tables	ii

Section 1 (S1): Model Preparation/Parameterization	1

S1.1 Model Parameterization	1

S1.1.1 Meteorological Inputs	1

S1.1.2 Simulation Domain Preparation	2

S1.1.3 Flow Model Preparation	3

S1.1.4 Transport Model Preparation	5

S1.1.4.1 Biosolids Application and Loading Rates	5

S1.1.4.2 Simulating the PFAS Source Term	6

S1.1.4.3 PFAS Interaction with Vadose Zone Soils	8

S1.1.4.4 Air-Water Interfacial Adsorption/Desorption	9

S1.1.5 Sources of Uncertainty in Model Predictions	10

S2: Supporting Material for Results Section (Section 3 in manuscript)	12

S3. Calculating Dilution Attenuation Factor	14

References	15

## List of Figures

Figure S1. Representative daily precipitation and potential evaporation data.	2

Figure S2. Hydrostratigraphy of the vadose zone for Site W and Site K.	3

Figure S3. Textural, hydraulic, and Aaw-θw characteristics of soils used in this evaluation.	5

Figure S4. Site K: Simulated PFAS desorption within the source zone for high and low desorption rate constants and BLR = 100 Mg/ha. .	12

Figure S5. Site W: Simulated PFAS desorption within the source zone for high and low desorption rate constants and LR= 100 Mg/ha and 5 Mg/ha. .	13

## List of Tables

Table S 1. Van Genucten model parameters and hydraulic properties of soils.	4

Table S 2. Regressions for PFAS soil concentration with loading rate.	6

Table S 3. Sorption and desorption rate parameters derived for the biosolids-amended surface soils.	7

Table S 4. Freundlich model parameters used to represent PFAS sorption in this work.	8

Table S 5. Sorption and desorption rate parameters derived for the vadose zone soils underlying the bio-amended surface soils.	9

Table S 6. Additional solute-specific model parameters.	10

Table S 7. Simulated Cw,max values resulting from different BLR values. These values were used in BLR-Cw,max linear regressions.	13

Table S 8. SBS,PFAS values calculated using the BLR-SBS,PFAS linear regressions.	14

Table S 9. Regression slopes from BLR-Cw,max SBS,PFAS-Cw,max linear regressions.	14

# Section 1 (S1): Model Preparation/Parameterization

## S1.1 Model Parameterization

The model used in this work was a version of the HYDRUS unsaturated flow and transport model (Šimůnek et al., 2016) that was recently modified to include key processes to simulate the transport of PFAS and other surface-active solutes (e.g., concentration-dependent surface tension-induced flows and nonlinear concentration-dependent air-water interfacial adsorption) within unsaturated soils. The modified model was described in detail in a recent open-access publication (Silva et al., 2020) and is currently undergoing additional modifications and testing as a part of an ongoing research project. The following sections detail the key HYDRUS model parameters used to simulate water flow and PFAS transport and how they were derived for this work. Key parameters include:

- Defining upper boundary characteristics (e.g., precipitation/evapotranspiration histories),

- Representing soil textural heterogeneity and assigning parameters to govern the flow of infiltrating water through the model domain (e.g., van Genuchten model parameters for each soil type and the saturated hydraulic conductivity, Ksat),

- Developing a relationship between biosolids loading rates (BLR) and mass/concentration of PFAS to establish representative initial concentration conditions for the source zone,

- Establishing an approach to represent the PFAS source term to govern rates of leaching from the source zone (here modeled as a two-site kinetic sorption/desorption condition),

- Representing sorption/desorption of PFAS within the soils/sediments underlying the surficial source, and

- Characterizing the degree of air-water interfacial adsorption for PFAS as surface-active solutes during transient flow conditions.

## S1.1.1 Meteorological Inputs

The Illinois sites represented in this work are located within the Humid Continental Climate zone (Köppen classification Dfa), with hot humid summers and cool to cold winters, where average annual precipitation totals range between 50 and 125 cm. Representative meteorological data was collected from a National Oceanographic and Atmospheric Administration (NOAA) weather station located at a civil/military airport in the area. Daily precipitation and evaporation data extending from 1999 to the end of 2020 were retrieved from the NOAA website. When daily evaporation totals were not provided, the Linacre model was applied to estimate the missing data (Linacre, 1977). An example of the temporal precipitation and evaporation data used in these simulations is provided in Figure S1. The meteorological dataset provided mean annual precipitation of 79.5 cm and an average daily evaporation potential of 0.16 cm/day. All precipitation was treated as rainfall, with no distinction made between rain and snow. Transpiration was not considered in the model. In some simulated cases, the available meteorological datasets needed to be extended into the future to resolve the complete transport of certain PFAS that were significantly retained in these systems. This was accomplished by repeating the original 15-year datasets multiple times to achieve the desired simulated duration.

Figure S1. Representative daily precipitation and potential evaporation data.

## S1.1.2 Simulation Domain Preparation

For the current evaluation, a one-dimensional (1D) simulation domain was prepared to simulate the vertical leaching of PFAS from a biosolid-amended surface source, as shown in Figure S2. The depth of the model domain was 10 feet (300 cm), which is consistent with the average depth of the water table at both test sites. The actual water table elevation has been observed to vary seasonally (e.g., Cravens et al., 1990; Roadcap et al., 1994), but the variation was generally less than 10% of the depth of groundwater. Therefore, a static representation of the average groundwater level was selected for simplicity. A time-variable atmospheric boundary condition was assigned to the top of the domain (z = 0 cm) to implement the meteorological data, and a constant pressure head (h) boundary was assigned at the base of the domain to represent a static water table (i.e., h = 0 at z = 300 cm).

Figure S2. Hydrostratigraphy of the vadose zone for Site W and Site K.

Two monitoring points were used in these simulations. The first was positioned just below the base of the source zone (30 cm) to monitor the dynamics of PFAS leaching from the source zone. The second monitoring point was positioned at the top of the capillary fringe at both sites (i.e., z = -283 cm selected for simplicity). As we are interested in evaluating the potential impacts of PFAS leaching from these biosolids sources and entering the groundwater, the top of the capillary fringe represents the first point of contact with the saturated zone.

## S1.1.3 Flow Model Preparation

The van Genuchten-Mualem model (van Genuchten, 1980) was selected to govern the relationship between soil moisture contents (θw) and pressure heads (h) (i.e., soil-water retention curves, SWRC) and the moisture dependence of the unsaturated hydraulic conductivity. van Genuchten (VG) parameters and other relevant soil-specific parameters representative of the various soil textural categories employed in this evaluation (i.e., saturated hydraulic conductivities (Ksat), dry bulk densities (ρb), and dispersivity values (αo)) are provided in Table S1.

Table S 1. Van Genucten model parameters and hydraulic properties of soils.

Surface soil textures varied between application sites. Hydraulically, Site W topsoil was modeled as a loam, and Site K soils were modeled as fine-grained sandy soil based on literature descriptions of the site (Higgins et al., 2011). It is acknowledged that hydraulic properties of biosolids-amended soils can vary depending on biosolid loading rate and time since initial application (e.g., Zare et al., 2010). However, parameterizing these effects would require data rarely available at most agricultural sites, and that is not available for Sites K and W.

Representative soil stratigraphic data representing the vadose zone beneath both sites was derived from engineers boring logs provided in the Illinois State Geological Survey’s Illinois Water Well (ILWATER) interactive map (https://isgs.illinois.edu/ilwater) at locations within the individual townships in which the sites are located (i.e., Florence township for Site W and Saint Anne Township for Site K). At Site W, the vadose zone possesses textural heterogeneity, and the Site K vadose zone does not (Figure S2). Site K is positioned within a zone of glacial drift deposits (Cravens et al., 1990) that generally consist of fine-grained and silty sand within the area.  Detailed measurements for soil hydraulic properties were not available for these sites.

VG parameters and hydraulic conductivities for the loam soils (Site W) were estimated using the Rosetta model (Schaap et al., 2001) within the HYDRUS-1D graphical user interface, assisted by sand, silt, and clay fractions derived from the standard USDA soil texture triangle as the centroid of the specific texture category boundary (Figure S3). For the sands (Sites W and K), representative VG parameters were derived from measured data presented by Baker (2001), as reported by Benson et al. (2014). Ksat values for the sands were estimated using the well-known Hazen equation (Terzaghi and Peck, 1996). SWRCs and the resulting Aaw-θw relationships are presented in Figure S3.

It should be noted that the soil hydraulic parameters (including n and a) predicted using pedotransfer functions such as Rosetta are associated with significant uncertainty when applying predicted values to a specific site soil. These uncertainties are discussed in manuscripts describing both the original Rosetta (Schaap et al., 2001) as well as its "new" version (Zhang and Schaap, 2017; 2019). For the purposes of this work, the use of texture-based representative VG parameters was considered reasonable and appropriate.

Figure S3. Textural, hydraulic, and Aaw-θw characteristics of soils used in this evaluation.

Finally, the flow model was conditioned by simulating infiltrating precipitation for the 5 years prior to biosolids application at both sites. This provided appropriate initial conditions for h, θw, and Kunsat. For Site K, the preconditioning period was April 1, 2000 - March 31, 2005. For Site W, the flow-model preconditioning period was October 1, 1999 – September 30, 2004. The start date of the simulation was based on the approximated month of the first biosolids application, which occurred in spring 2005 for Site K and fall 2004 for Site W (Higgins et al., 2011).

## S1.1.4 Transport Model Preparation

For the transport model, a concentration flux boundary was applied to the top of the domain, and a zero-concentration gradient boundary was assigned to the base of the model domain. The PFAS source zone was assigned to the top 30 cm of the simulation domain (see Figure S2), representing applied biosolids that have been mechanically tilled into the soil. Tilling of the biosolids-amended soil was assumed to provide a uniform and homogenous medium, which has been shown to be reasonable for re-worked agricultural soils (Peterson et al., 1997; Gjettermann et al., 1997). There is potential for the development of preferential flow effects over the timescale of these simulations, but it is inherently difficult to accurately characterize the degree and scale of these effects as a part of model parameterization. This will be true of many agricultural sites. Therefore, the assumption of flow and transport homogeneity within modeled soil profile layers was selected and was considered representative for the purpose of this evaluation for the source zone and for the additional strata underlying the source zone.

## S1.1.4.1 Biosolids Application and Loading Rates

Biosolids loading rates (BLR) at Site K and Site W ranged between 5.6 to 36.3 Mg/ha and 11.1 to 44.5 Mg/ha (Higgins et al., 2011), respectively, across multiple adjacent test plots. As the average agronomic rate of application is 10 US dry tons per acre (or 22.5 Mg/ha) (WEF, 2017), it was assumed that the maximum BLRs at each site represent the allowable yearly agronomic rate for that site based on nutrient loading and metals contents in accordance with USEPA Part 503 Rule. Sepulvado et al. (2011) demonstrated that the PFAS source strength (i.e., the mass of PFAS per unit mass of biosolids-amended topsoil) for the biosolids used at these sites could be represented as a linearly increasing function of the cumulative BLR as:

SBS,PFAS=m BLR										     [S1]

where SBS,PFAS is the biosolids-amended soil concentration of PFAS (ng/gsoil), m is the slope of the linear regression, and BLR has units of Mg/ha. A subset of the regression equations provided by Sepulvado et al. (2011) is represented here in Table S2 for the PFAS of interest in this work. These relationships were used to scale the strength of the PFAS source within biosolids-amended surface soils for individual simulations.

Table S 2. Regressions for PFAS soil concentration with loading rate (from Sepulvado et al., 2011).

At each site, biosolids were applied annually for three consecutive years (Higgins et al., 2011), assumed here to begin in October 2004 at Site W and April 2005 for Site K. Biosolids were applied to the Site W test plots in the fall of 2004, 2005, and 2006. Biosolids were applied to the Site K test plots in the Spring of 2005, 2006, and 2007. Simulations were performed in accordance with this biosolids application schedule. However, simulations were additionally, and initially, performed for a one-time application event to isolate the effects of BLR on the rates of loss of PFAS mass from the source zone and the impacts on groundwater.

## S1.1.4.2 Simulating the PFAS Source Term

The release of PFAS from the source media (i.e., the biosolids amended surface soil), and the exchange of PFAS mass within the source media, was simulated using the two-site kinetic sorption/desorption model. The two-site kinetic model assumes sorption sites can be divided into two fractions: Type-1 sites where instantaneous and local equilibrium adsorption/desorption occurs and Type-2 sites where sorption is rate-limited and can be described by first-order kinetics. Additional details of the two-site model have been provided elsewhere (e.g., Šimůnek and van Genuchten, 2008). Therefore, the source term is modeled here as a temporally diminishing source of PFAS once biosolids are applied to the land surface. While in situ production of PFAS within biosolids and biosolids-amended soils due to precursor transformation reactions has been demonstrated in mesocosm leaching experiments (Venkatesan, and Halden, 2014; Schaefer et al., 2022), production rates are PFAS-specific and have not been rigorously quantified for the PFAS of interest here. Further, the observed significant PFAS interaction with the solid phases within the biosolids-amended source soils will ultimately govern the overall rate of PFAS leaching regardless of the production of more PFAS mass.

Batch experiments to determine PFAS desorption soil-water distribution coefficients (Kdes) for biosolid-amended soils collected from the test sites were performed as a part of the research described by Sepulvado et al. (2011) but were not included in that publication. The results of these experiments are presented here in Table S3. To provide a better comparison between sites, the averaged Kdes values were used in these simulations. Linear PFAS sorption/desorption was assumed for the biosolids-amended soils.

Table S 3. Equilibrium desorption coefficients and nonequilibrium rate coefficients derived for the biosolids-amended surface soils.

The data presented by Sepulvado et al. (2011) also demonstrated the potential for rate-limited desorption from the biosolids-amended soils for some or all PFAS investigated, an observation that is consistent with disappearance rate constants for PFAS presented by Washington et al. (2010) for biosolids-amended soils collected from a site in Alabama. Desorption rate experiments were additionally performed by Sepulvado et al. (2011) using biosolids-amended soils collected from the Illinois sites but were not presented in that publication. This unpublished data generally showed rapid initial desorption (within the first 1 to 1.5 days) and slower desorption after that, which is consistent with the two-site kinetic sorption/desorption model. The fraction of Type-1 sites for the source media was derived from the desorption rate experiments as being 0.84 ± 0.09 (i.e., a fraction of Type 2 sites was then established as 0.16). Unfortunately, the Sepulvado et al. (2011) desorption experiments were not performed over a sufficient duration to capture the true slow desorption rates associated with these biosolids-amended soils.

As such, the rate constants presented by Washington et al. (2010) were used in these simulations (Table S3). The Washington et al. (2010) rate dataset was selected because it was developed from the analysis of individual biosolids-amended soil samples collected over several years after application. It, therefore, provides rates that are more representative over the timescale of these simulations.  Given the site-specific uncertainty in the PFAS rate coefficients, the Washington et al. (2010) dataset was also used as the basis for a sensitivity analysis on the magnitude of the slow desorption rate coefficient, with high- and low-rate coefficient cases (see Table S3) representing a factor of 10 increase and decrease in the measured values, respectively. Sorption and rate parameters presented in Table S3 are used to govern the release of PFAS from the PFAS source zone. Sorption and desorption of PFAS were assumed to be governed by the same rate coefficient (i.e., no sorption/desorption hysteresis). It is important to note that desorption rate coefficients presented by Washington et al. (2010) are consistent with values recently presented by Schaeffer et al. (2021) for PFAS desorption from aqueous film-forming foam (AFFF) contaminated soils.

## S1.1.4.3 PFAS Interaction with Vadose Zone Soils

For the vadose zone soils (i.e., those underlying the surficial biosolids-amended soils modeled as the PFAS source at both sites), site samples were not collected to characterize PFAS sorption/desorption. To parameterize sorption/desorption for these soils, representative parameters were derived from the results of Guelfo and Higgins (2013) and Guelfo et al. (2020), who investigated PFAS transport in soils similar in texture to those present beneath both test sites. The data was initially modeled using the Freundlich equation, as shown in Table S4, and some mild to moderate nonlinear sorption behavior was observed in some cases. However, when the Freundlich model fit to the data was extrapolated to the concentration range of interest here (i.e., ng/g for solid phase; ng/L for aqueous phase), the Freundlich exponents (n) approach unity in many cases. Here, solutes with n greater than unity were treated as linearly sorbing solutes with Kf becoming the linear distribution coefficient (Kd) and n assigned a value of 1. For n < 1, Kf and n were used as they appear in Table S4.

Table S 4. Freundlich model parameters used to represent PFAS sorption in this work.

In addition to nonlinear sorption, recent 1D transport experiments have demonstrated that PFAS transport can also be nonideal, with rate-limited processes contributing to PFAS retention (e.g., Brusseau et al., 2019; Brusseau, 2020; Guelfo et al., 2020).  To account for nonideal transport, the two-site kinetic sorption model was again used for the underlying vadose zone soils. Fractions of equilibrium sites (F) (Type-1 sites) and rate coefficients for the slow sorption/desorption sites (Type-2 sites) were additionally derived from the work of Guelfo et al. (2020) using the PFAS column transport results. In HYDRUS, F = F1, as defined in the main text and F2 is not required as input as it is calculated as: F2 = F - F1. As a soil-specific parameter in HYDRUS, F was assigned a value of 0.3 based on the interpretation of the data described by Guelfo et al. (2020). F was assigned a unit value when kinetic behavior was not observed. When kinetic processes were observed (i.e., early breakthrough and breakthrough curve tailing), a value of 0.003 day-1 was assigned as the rate constant for that soil. For PFAS, for which equilibrium transport was observed, the rate constant was assigned an elevated value of 10 day-1 to offset the effects of rate-limited sorption provided by the assignment of F = 0.3 in these cases: the net effect resulting in equilibrium transport behavior. The actual kinetic parameter assignment for each PFAS within the vadose zone soils is provided in Table S5. As in biosolids-amended soils, soil sorption and desorption of PFAS in underlying vadose zone soils were assumed to have the same rate coefficient.

Table S 5. Sorption and desorption rate parameters derived for the vadose zone soils underlying the bio-amended surface soils.

## S1.1.4.4 Air-Water Interfacial Adsorption/Desorption

PFAS adsorption at air-water interfaces within the vadose zone provides an additional source of retention during unsaturated transport (i.e., leaching). As described elsewhere (e.g., Silva et al., 2019; Silva et al., 2021), the degree of PFAS air-water interfacial adsorption is dependent on the affinity of the PFAS for the interface and the area of the AWI (Aaw) within the water-unsaturated porous medium. The magnitude of the Aaw is dependent on the soil moisture content (θw) and on textural characteristics of the soil (e.g., particle size, distribution, surface texture). For the modified HYDRUS model, the dependence of Aaw on Θw is calculated directly from SWRCs via integration, using the approach described by Bradford and Leij (1997) and Bradford et al. (2015) as:

AawΘw= 1γoΘwΘsPawΘwdΘw=ρwgγoΘwΘshΘwdΘw 					     [S2]

where Paw is the capillary pressure (ML-1T-2), γo is the air-water surface tension (MT-2), θs is the saturated water content (-), ρw is the density of water (ML-3), g is the gravitational acceleration (LT-2), and h is the pressure head (L). The magnitude of Aaw increases with decreasing Θw as Θw approaches Θr. The incremental change in the magnitude of the Aaw is linked to the thermodynamic work required to form the interface as Θw changes, as first proposed by Leverett (1941). The application of this approach is demonstrated in Figure S3. The use of Equation S2 to predict Aaw-Θw relationships for soils was discussed in detail in a previous publication (i.e., Silva et al., 2022).

The degree of AWI adsorption of PFAS is also non-linearly dependent on aqueous concentration (Silva et al., 2019). The modified HYDRUS model allows AWI adsorption to be modeled using linear, Freundlich, and Langmuir adsorption models. Herein, the Langmuir model is used. The dependence of the interfacial adsorbed concentration (Γ) on the solution concentration (Cw) of PFAS under Langmuir adsorption is modeled as:

ΓCw=ΓmaxKL,awCw1+KL,awCw										     [S3]

where Γmax is the maximum surface concentration and KL,aw is the Langmuir coefficient for AWI adsorption, both of which are derived from fitting Equation S3 to measured surface concentration isotherm data. AWI adsorption is considered instantaneous and reversible, in keeping with the assumptions of the Langmuir model. KL,aw and Γmax values for each PFAS solute considered in this work are provided in Table S6.

It is important to note that the Freundlich isotherm has also been suggested as an appropriate model for describing AWI adsorption, especially as PFAS concentrations become increasingly dilute (Schaefer et al., 2019). Adopting the Freundlich model implies that AWI adsorption increases with decreasing concentration. However, the Freundlich model does not assume a maximum for AWI adsorption, as does the Langmuir model, and therefore should be less representative for AWI adsorption as concentrations increase. Silva et al. (2021) proposed a coupled Langmuir-Freundlich model to account for AWI adsorption at both low and high PFAS concentrations to resolve this discrepancy. However, the representativeness of the Freundlich model over the Langmuir model for low concentration PFAS AWI adsorption is a topic of ongoing debate. Although the current PFAS-modified version of HYDRUS allows AWI adsorption to be modeled using either Langmuir or Freundlich isotherms, the Langmuir model was selected here, largely because the applicability of the Freundlich model has not yet been validated for transport in porous media. However, if the Freundlich model were used in these simulations, AWI adsorption would generally play a larger role in limiting the rate of PFAS leaching to the saturated zone.

Table S 6. Additional solute-specific model parameters.

## S1.1.5 Sources of Uncertainty in Model Predictions

While Kd values for the biosolids-amended source zone soils were derived from laboratory measurements using actual site soil, Kd values used in these simulations for the underlying vadose zone soils were from experimental data on soils that shared similar textural characteristics. For PFAS, similarities in the textural classification do not necessarily mean similarities in sorption characteristics. However, the values used for the PFAS evaluated are reasonably similar to those reported elsewhere in the literature for these soil types (e.g., Schaefer et al., 2021) and, therefore, should not represent a large error, particularly over a long period of simulation.

Preferential water flow and its effects on PFAS transport were also not considered in these simulations. Preferential flow (e.g., macroporous and/or fracture flow) is common in agricultural surface soils and can enhance the rate of transport of contaminants through the surficial vadose zone. However, properly characterizing preferential flow effects for the purpose of simulations at the field plot scale can be challenging given the heterogeneous nature of the process. Additionally, this type of characterization was not performed for these sites. Therefore, rather than introducing a nonideal flow process with an unknown contribution effect, we opted to model flow as matrix flow. Further, the surficial soils at these sites were tilled annually to incorporate biosolids, and the assumption that preferential flow pathways were effectively destroyed in the process was considered a good one.

Additional factors that were not considered in these simulations include: (1) the effect of snowpack accumulation and melting as part of the precipitation input, (2) flow effects and the potential for the loss of PFAS from plant uptake in the soil zone, (4) the effect of a dynamic water table over the simulation period, and (5) the effect of lateral flow within the heterogeneous Site W vadose zone. Each of these can impact PFAS transport in the vadose zone. For example, treating all precipitation as rainfall and excluding snowpack accumulation during winter months likely results in an underestimation of transport times. Likewise, excluding transpiration flux out of the simulation domain would have the effect of removing infiltrating water from the simulation, which would also ultimately underestimate transport times. The seasonal rise and fall of the water table elevation varies the length-scale for transport and influences the distribution of soil moisture. During the wet seasons, the rise of the water table could allow PFAS leaching through the vadose zone to contact the saturated zone sooner relative to the stationary water table considered here. Finally, the 1D model used here does not account for the lateral flow of water in the vadose zone, which generally results in an underprediction of transport times. At Site W, lateral flow at points of contact between soil layers can reduce the rate of vertical flow. By excluding lateral flow, the arrival of PFAS at the saturated zone likely occurs sooner in these simulations. Collectively eliminating these processes from the current model yields conservative estimates of PFAS transport (i.e., shorter transport times) from biosolids through the vadose zone and contacting the saturated zone. However, the current modeling approach does provide a practical base simulated condition for use by regulators and others seeking conservative estimates of risk to groundwater associated with PFAS in biosolids.

# S2: Supporting Material for Results Section (Section 3 in manuscript)

Figure S4. Simulated PFAS desorption within the source zone of Site K for high and low desorption rate constants and BLR = 100 Mg/ha and 5 Mg/ha. The values of the high and low desorption rate constants used are provided in Table S3. Note that year 15 is the calendar year 2020.

Figure S5. Simulated PFAS desorption within the source zone of Site W for high and low desorption rate constants and BLR= 100 Mg/ha and 5 Mg/ha. The values of high and low desorption rate constants used were provided in Table S3. Note that year 15 in these plots is the calendar year 2020.

Table S 7. Simulated Cw,max values resulting from different BLR values. These values were used in BLR-Cw,max linear regressions.

Table S 8. SBS,PFAS values calculated using the BLR-SBS,PFAS linear regressions.

Table S 9. Regression slopes from BLR-Cw,max SBS,PFAS-Cw,max linear regressions.

# S3. Calculating Dilution Attenuation Factor

PFAS considered here do not decay significantly, unlike most hydrocarbon contaminants, and therefore Dilution-Attenuation Factors must be determined using analytical solutions that do not include decay. From Domenico and Schwartz (1990):

Cx,y,z,tCo=1/8 expx2αx1-1+4λαxverfcx-vt1+4λaxv2αxvterfy+Y22αyx-erfy-Y22αyxerfz+δgw2αzx-erfz-δgw2αzx						     [S4]

where:

C = Dissolved-phase concentration (ML-3)

Co = Dissolved-phase concentration at the source (ML-3)

v = Retarded solute velocity (LT-1)

αx = Longitudinal dispersivity (L)

αy = Lateral dispersivity (L)

αz = Vertical dispersivity (L)

x,y,z = Spatial coordinates (L)

t = time (T)

x = Distance along the centerline from down-gradient edge of dissolved plume source (L)

Y = Width of the soil source area perpendicular to the groundwater flow direction (L)

δgw = Groundwater mixing zone thickness (L)

DAF = Dilution attenuation factor (-) = Co/C

At the centerline DAF can be determined by assuming y = 0, z = 0, and x << vt as:

CCo=1DAF=12 erfcx-vt2αxvterfY4αyxerfδgw2αzx				     [S5]

and at steady state, or x << vt, DAF is dependent on mechanical mixing alone, and Equation S5 reduces to:

1DAF=erfY4αyxerfδgw2αzx.				                 			     [S6]

# References

Baker, K. (2001), Investigation of Direct and Indirect Hydraulic Property Laboratory Characterization Methods for Heterogeneous Alluvial Deposits: Application to the Sandia- Tech Vadose Zone Infiltration Test Site, MS Thesis, New Mexico Institute of Mining and Technology, Socorro, NM.

Benson, C.H., Chiang, I., Chalermyanont, T., Sawangsuriya, A. (2014). Estimating van Genuchten parameters α and n for clean sands from particle size distribution data. Geotechnical Special Publication No. 233, From Soil Behavior Fundamental to Innovations in Geotechnical Engineering. American Society of Civil Engineers.

Bradford, S.A., Leij, F.J. (1997). Estimating interfacial areas for multi-fluid soil systems. J. Contam. Hydrol. 27:83–106.

Bradford, S.A., Wang, Y., Torkzaban, S., Šimůnek, J. (2015). Modeling the release of E. coli D21g with transients in water content. Water Resour. Res., 51, 3303-3316.

Brusseau, M.L., 2020. Simulating PFAS transport influenced by rate-limited multi-process retention. Water Res., 168, 115179.

Brusseau, M.L., Khan, N., Wang, Y., Yan, N., Van Glubt, S. Carroll, K.C. 2019. Non-ideal transport and extended elution tailing of PFOS in soil. Environ. Sci. Technol., 53, 10654-10664.

Cravens, S.J., Wilson, S.D., Barry, R.C., 1990. Regional Assessment of the Ground-Water Resources in Eastern Kankakee and Northern Iroquis Counties. Illinois State Water Survey, Champaign IL, Report of Investigation 111.

Domenico, P.A., Schwartz, F.W., 1990. Physical and Chemical Hydrogeology. John Wiley & Sons, New York.

Gjettermann, B, Nielsen, K.L., Petersen, C.T., Jensen, H.E., Hansen, S., 1997. Preferential flow in sandy loam soils as affected by irrigation intensity. Soil Technol., 11(2), 139-152.

Guelfo, J.L., Higgins, C.P., 2013. Subsurface transport potential of perfluoroalkyl acids at aqueous film-forming foam (AFFF)-impacted sites. Environ. Sci. Technol. 47, 4164-4171.

Guelfo, J.L., Wunsch, A., McCray, J., Stults, J.F., Higgins, C.P., 2020. Subsurface transport potential of perfluoroalkyl acids (PFAAs): Column experiments and modeling., J. Contam. Hydrol., 233, 103661.

Higgins, C.P., Paesani, Z.J., Chalew, T.E.A., Halden, R.U., Hundal, L.S., 2011. Persistence of triclocarban and triclosan in soils after land application of biosolids and bioaccumulation in Eisenia foetida. Environ. Toxicol. Chem., 30(3), 556-563.

Leverett, M.C., 1941. Capillary behavior in porous solids, Tans. Am. Inst. Min. Merall. Pet. Eng., 142, 152-169.

Linacre, E.T., 1977. A simple formula for estimating evaporation rates in various climates, using temperature data alone. Agric. Meteorol., 18(6), 409-424.

Petersen, C.T., Hansen, S., Jensen, H.E., 1997. Depth distribution of preferential flow patterns in a sandy loam soil as affected by tillage. Hydrol. Earth Syst. Sci., 4, 769-776.

Roadcap, G.S., Cravens, S.J., Smith, E.C., 1994. Meeting the Growing Demand for Water: An Evaluation of the Shallow Ground-Water Resources in Will and Southern Cook Counties, Illinois. Illinois State Water Survey, Research Report 123.

Schaap, M.G., Leij, F.J., van Genuchten, M. Th., 2001. Rosetta: a computer program for estimating soil hydraulic parameters with hierarchical pedotransfer functions. Journal of Hydrology 251, 163–176.

Schaefer, C.E., Culina, V., Nguyen, D., Field, J., 2019. Uptake of poly- and perfluoroalkyl substances at the air-water interface. Environ. Sci. Technol., 53 (21), 12442-12448.

Schaefer, C.E., Nguyen, D., Christie, E., Shae, Stefanie, Higgins, C.P., Field, J.A., 2021. Desorption of poly-and perfluoroalkl substances from soil historically impacted with aqueous film-forming foam. J. Environ. Eng., 147 (2), 06020006-1. DOI: 10.1061/(ASCE)EE.1943-7870.0001846.

Sepulvado, J.G., Blaine, A.C., Hundal, L.S., Higgins, C.P., 2011. Occurrence and fate of perfluorochemicals in soil following the land-application of municipal biosolids. Environ. Sci. Technol., 45(19), 8106-8112.

Silva, J.A.K., W.A. Martin, J.L. Johnson, McCray, J.E., 2019. Evaluating Air-Water and NAPL-Water Interfacial Adsorption and Retention of Perfluorocarboxylic Acids with the Vadose Zone. J. Contam. Hydrol., 223, 103472. https://doi.org/10.1016/j.jconhyd.2019.03.004

Silva, J.A.K., Šimůnek, J., McCray, J.E., 2020. A modified HYDRUS model for simulating PFAS transport in the vadose zone. Water, 12, 2758; https://doi:10.3390/w12102758.

Silva, J.A.K., Martin, W. A., McCray, J.E., 2021. Air-water interfacial adsorption coefficients for PFAS when present as a multi-component mixture, J. Contam. Hydrol., 236, 103731.

Silva, J.A.K., Šimůnek, J., McCray, J.E., 2022. Comparison of methods to estimate air-water interfacial areas for evaluating PFAS transport in the vadose zone. J. Contam. Hydrol., 247, 103984.

Šimůnek, J., van Genuchten, M.Th. (2008). Modeling nonequilibrium flow and transport processes using HYDRUS. Vadose Zone J., 7(2), 782-797.

Šimůnek, J., van Genuchten, M. Th., Šejna, M., 2016. Recent developments and applications of the HYDRUS computer software packages. Vadose Zone Journal, 15(7), pp. 25, doi: 10.2136/vzj2016.04.0033.

Terzaghi, K., Peck, R.B., 1996. Fundamentals of Soil Mechanics, Wiley, New York.

van Genuchten, M.Th., 1980. A closed-form equation for predicting the hydraulic conductivity of unsaturated soils. Soil Sci. Soc. Am. J., 44, 892-898.

Washington, J.W., Yoo, H., Ellington, J.J., Jenkins, T.M., Libelo, E.L. (2010). Concentrations, distribution, and persistence of perfluoroalkylates in sludge-applied soils near Decatur, Alabama, USA. Environ. Sci. Technol., 44(22), 8390-8396.

Zare, M., Afyuni, M., Abbaspour, K.C., 2010. Effects of biosolids application on temporal variations in soil physical and unsaturated hydraulic properties, J. Residuals Sci. Technol., 7(4), 227-235.

Zhang, Y., Schaap, M.G. (2017). Weighted recalibration of the Rosetta pedotransfer model with improved estimates of hydraulic parameter distributions and summary statistics (Rosetta3). J. Hydrol., 547, 39-53.

Zhang, Y., Schaap, M.G. (2019). Estimation of saturated hydraulic conductivity with pedotransfer functions: A review. J. Hydrol., 575, 1011-1030.